Mini pupper

Forward & inverse kinematics and jacobian

# computation of Forward kinematics using Denavit–Hartenberg parameters

## Fixed leg Reference coordinate frame #0

- Right-hand coordinate system

- Centred on coxa revolute joint

- X upward

- Y leftward

- Z backward

## DH Parameters

- θ is rotation about previous Z axis, from old X to new X

- d is offset along previous Z axis

- a is offset along new X axis

- α is rotation about new X, from old Z to new Z

| Links | θ | d | a | α |
| --- | --- | --- | --- | --- |
| 1 | θc | 0 | 0 | -90° |
| 2 | θh+90 | Lc (Left Legs) -Lc (Right Legs) | Lf | 0 |
| 3 | θk-θh | 0 | Lt | 0 |

| Variables | Description | Values |
| --- | --- | --- |
| Lc | Distance from coxa revolute joint to hips revolute joint along X1 | 26 mm |
| Lf | Distance from hips revolute joint to knee revolute joint along X2 | 50 mm |
| Lt | Distance from knee revolute joint to foot along X3 | 60 mm |
| θc | Coxa revolute joint angle, from X0 axis to X1 axis | controlled by servo #1 |
| θh | Hips revolute joint angle, from body HORIZONTAL to FEMUR (X2) | controlled by servo #2 |
| θk | Knee revolute joint angle, from body HORIZONTAL to TIBIA (X3) | controlled by servo #3 |

## Transformation matrices using DH parameters

Coordinate frame of link i, noted Oi, is located at the end of the link, aligned with new X and Z axis, after all rotations and translations.

### (reminder) DH transformation matrix

Transformation from coordinate frame j to coordinate frame j-1:

Tjj-1θjdjajαj=cosθj-sinθjcosαjsinθjsinαjajcosθjsinθjcosθjcosαj-cosθjsinαjajsinθj0sinαjcosαjdj0001

### COXA (j=1)

T10=cc0-sc0sc0cc00-1000001

where

| cc | cosθc |
| --- | --- |
| sc | sinθc |

### Femur (j=2)

- For left legs:

T21=ch-sh0Lfchshch0Lfsh001Lc0001

- For right legs:

T21=ch-sh0Lfchshch0Lfsh001-Lc0001

where

| ch | cosθh+90 |
| --- | --- |
| sh | sinθh+90 |

### TIBIA (j=3)

T32=ckh-skh0Ltckhskhckh0Ltskh00100001

where

| ckh | cosθk-θh |
| --- | --- |
| skh | sinθk-θh |

### « end-effector » Transformation matrix to leg reference frame #0

- For left legs:

T30=T10T21T32=ccck-ccsk-scLtccck+Lfccch-Lcscscck-scskccLtscck +Lfscch+Lccc-sk-ck0-Ltsk-Lfsh0001

- For right legs:

T30=T10T21T32=ccck-ccsk-scLtccck+Lfccch+Lcscscck-scskccLtscck +Lfscch-Lccc-sk-ck0-Ltsk-Lfsh0001

where

| ck | cosθk+90 | ckhch-skhsh |
| --- | --- | --- |
| sk | sinθk+90 | skhch+ckhsh |

## Transformation matrices using modified DH parameters

Coordinate frame of link i, noted Oi, is located the beginning of the link, aligned with previous Z axis and new X axis, after rotation θi and translation di.

### (reminder) modified DH transformation matrix

Transformation from coordinate frame j to coordinate frame j-1:

Tjj-1θjdjaj-1αj-1=cosθj-sinθj0ajsinθjcosαj-1cosθjcosαj-1-sinαj-1-djsinαj-1sinθjsinαj-1cosθjsinαj-1cosαj-1djcosαj-10001

### COXA (j=1)

T10=cc-sc00sccc0000100001

where

| cc | cosθc |
| --- | --- |
| sc | sinθc |

### Femur (j=2)

- For left legs:

T21=ch-sh00001Lc-sh-ch000001

- For right legs:

T21=ch-sh00001-Lc-sh-ch000001

where

| ch | cosθh+90 |
| --- | --- |
| sh | sinθh+90 |

### TIBIA (j=3)

T32=ckh-skh0Lfskhckh0000100001

where

| ckh | cosθk-θh |
| --- | --- |
| skh | sinθk-θh |

### FOOT (j=4)

T43=100Lt010000100001

### « end-effector » Transformation matrix to leg reference frame #0

- For left legs:

T40=T10T21T32T43=ccck-ccsk-scLtccck+Lfccch-Lcscscck-scskccLtscck +Lfscch+Lccc-sk-ck0-Ltsk-Lfsh0001

- For right legs:

T40=T10T21T32T43=ccck-ccsk-scLtccck+Lfccch+Lcscscck-scskccLtscck +Lfscch-Lccc-sk-ck0-Ltsk-Lfsh0001

where

| ck | cosθk+90 | ckhch-skhsh |
| --- | --- | --- |
| sk | sinθk+90 | skhch+ckhsh |

Note: both methods (DH and Modified DH) give the same result.

## Forward Kinematic

Position of foot in leg reference frame #0 is given by the 3x1 submatrix of T30 describing translation:

- For left legs:

Pθc,θh,θk0=Ltccck+Lfccch-LcscLtscck +Lfscch+Lccc-Ltsk-Lfsh

- For right legs:

Pθc,θh,θk0=Ltccck+Lfccch+LcscLtscck +Lfscch-Lccc-Ltsk-Lfsh

# computation of jacobian

Velocity of the foot in leg reference frame #0 is given by the Jacobian J :

P0= Jθ×θ 		where 		θ=θcθhθk

## Partial Differentiation Method

J=dPxdθcdPxdθhdPxdθkdPydθcdPydθhdPydθkdPzdθcdPzdθhdPzdθk

- For left legs:

J=-Ltscck-Lfscch-Lccc-Lfccsh-LtccskLtccck +Lfccch-Lcsc-Lfscsh-Ltscsk 0-Lfch-Ltck

- For right legs:

J=-Ltscck-Lfscch+Lccc-Lfccsh-LtccskLtccck +Lfccch+Lcsc-Lfscsh-Ltscsk 0-Lfch-Ltck

## Velocity propagation method

### Velocity propagation (Theory)

i+1ωi+1=Rii+1.iωi+θi+1i+1Zi+1

i+1vi+1=Rii+1.ivi+iωi⋀ipi+1

0ω0=0

0v0=0

### COXA (i=1)

1ω1=R01.0ω0+θ11Z1

i1v1=R01.0v0+0ω0⋀0p1

then

1ω1=θc1Z1=00θc

i1v1=0

### HIPS (i=2)

2ω2=R12.1ω1+θ22Z2

2v2=R12.1v1+1ω1⋀1p2

where

T21=ch-sh00001Lc-sh-ch000001

then

2ω2=-shθc-chθcθh

2v2=-LcchθcLcshθc0

### KNEE (i=3)

3ω3=R23.2ω2+θ33Z3

3v3=R23.2v2+2ω2⋀2p3

where

T32=ckh-skh0Lfskhckh0000100001

then

3ω3=-skθc-ckθcθh+θkh=-skθc-ckθcθk

3v3=-ckLcθc+skhLfθhskLcθc+ckhLfθhLfchθc

### tibia/FOOT (i=4)

4ω4=R34.3ω3

4v4=R34.3v3+3ω3⋀3p4

where

T43=100Lt010000100001

then

4ω4=-skθc-ckθcθk

4v4=-ckLcθc+skhLfθhskLcθc+ckhLfθh+LtθkLfchθc+Ltckθc

### Jacobian in last frame

Extracting θ vector:

4v4=-ckLcskhLf0skLcckhLfLtLfch+Ltck00θcθhθk

then

4J=-ckLcskhLf0skLcckhLfLtLfch+Ltck00

### Jacobian in base frame

0J=40R4J

where

R40=ccck-ccsk-scscck-scskcc-sk-ck0

then

J=-Ltscck-Lfscch-Lccc-Lfccsh-LtccskLtccck +Lfccch-Lcsc-Lfscsh-Ltscsk 0-Lfch-Ltck

Note: both methods (partial differentiation and velocity propagation) give the same result.

# computation of ground reaction force

Ground reaction force in leg reference frame #0 is given by the inverse transposed Jacobian Jand the torque of each joint:

GRF0=-F0=- JT-1τ 		where 		τ=τcτhτk

## Singularities

Numerical approach may be used for singularities computation

## INVERSE maTRIX (if NON-ZERO determinant)

Numerical approach may be used for inverse Jacobian matrix computation

# Inverse KINEMATIC

Geometric approach.